Dear Users,

Since function setting is quite professional, please read this user’s manual carefully before your operation.

H.264

Network Video Decoder

User’s Manual

(With remote control and VGA output)

Release date:   2010/04/02

Revision:   1.0

Content

1. Introduction	2

1.1 Purpose of establishment	2

1.2 Range of Application	3

2．Product Introduction	4

2.1 Brief Introduction	4

2.2 Technical Specification	4

3.How-to-use	5

3.1 Packing List	5

3.2Installation and attention	5

3.3 Connecting Diagram	6

3.4.1 Searching device	6

3.4.2 Adding decoder	8

3.4.3 Decoder Param setting	10

3.4.4 Decoder Channel Param Setting	11

3.4.5 Alarm Param Setting	12

3.4.6 Audio Control and Talk Control	12

3.4.7 Loop View Setting	13

3.4.8 PTZ control	13

3.4.9 Tools	14

3.4.10 Use the software IMagineWordDMatrix for upgrading	16

3.5 Operation of dummy keyboard ( Remote Controller)	17

3.5.1 Instructions of main menu	17

3.5.2 View mode and view mode interface	18

3.5.3 View On menu	18

3.5.4 Address book menu. (This function is not realized yet.)	19

3.5.5 Loop connect menu	19

3.5.6 Loop setting menu	19

3.5.7 Interface of Add Server	20

3.5.8 Start and stop the audio	21

3.5.9 Start and stop Talk	21

3.5.10 System setting menu interface	21

3.5.11 Network setting menu interface	22

3.5.12 System Param menu interface	23

3.5.13 Video Param Interface  (VGA Param)	24

3.5.14 Audio Param setting interface	25

3.5.15 PTZ control menu	25

3.5.16 Alarm status menu interface	26

3.5.17 Alarm setting menu interface	27

4.	Frequent Asked Questions	28

Appendix Dome Camera Keyboard Application	29

# 1. Introduction

## 1.1 Purpose of establishment

This manual is established for businessmen and end users to understand functions, design program and how-to-use of this product.

## 1.2 Range of Application

Users: End users, businessmen, and technical support personnel

Contents: introduction, performance parameters and operation instructions of Enhanced H.264 Network Video Decoder

Models: Enhanced One-way H.264 Decoder and Enhanced 4-way H.264 Decoder.

# 2．Product Introduction

## 2.1 Brief Introduction

Thank you for choosing our product! We will provide our best service to you.

Enhanced One-way H.264 Decoder and Enhanced 4-way H.264 Decoder can directly output digital A/V data to TV wall after receiving and decoding from internet without PC platform; and meanwhile, it can realize phonetic Talk with decoder. Being equipped with embedded GUI, it can support keyboard operation. Operation of such models is easy and convenient.

(Remark: TV wall screen is black when connecting starts, since there is no LOGO picture.)

## 2.2 Technical Specification

| Video format | NTSC or PAL |
| --- | --- |
| Audio compression | G.722 and G.711 |
| Delay time | Less than 200 millisecond(LAN) |
| Resolution | Support D1:704*576 optimal, compatible Half D1:704*288, CIF:352*288, QVGA:320*240 |
| VGA output | Resolution: 800*600_50HZ 1024*768_60HZ,1280*720_60HZ,1920*1080_60HZ |
| IO & control interface | RS485 transparent channel, RS232 |
| Audio input | One-way mic audio input |
| Audio output | 3.5mm one-way audio output |
| BNC audio input | Audio line-in |
| BNC audio output | Audio output |
| Hardware | CPU: Dedicated high-performance multimedia processor Hi 3511 OS: embedded Linux operation system 128MB DDR2 memory, flash 8MB |
| Video stream | Analog video output |
| Audio | Two-way audio |
| Video compression | H.264 |
| Embedded GUI | Intuitive and convenient operation |
| Input method support | Convenient for input domain name input |
| Multi-server Loop viewing | Supporting channel select and programmable control of channel inspection |
| Network interface | 10/100Mbps RJ45 |
| Temperature | -25-80°C |
| Humidity | 20-80% |
| Input voltage | DC12V/2A |

# 3. How-to-use

## 3.1 Packing List

| Name | Quantity |
| --- | --- |
| Video decoder host | 1 pc |
| DC12V/2A adaptor | 1 set |
| Users’ Manual for Network Video Server Encoder/ Decoder | 1 pc |
| Corollary CD | 1 pc |
| Certification and guarantee card | 1 pc each |

## 3.2Installation and attention

## 3.3 Connecting Diagram

.3.4 Setting decoder

3.4.1 Searching device

1) IP address is 192.168.1.19 and subnet mask is 255.255.255.0 for each network video decoder when delivery.

2) Test if the network video decoder starts up normally. Operate as per sequence of <Start →run →command> under Windows system. Open the command-line window, and type “<ping 192.168.1.19>” into the command-line window.

3) You can use the supporting program in the CD named Camera Search to determine the IP address of the network video decoder. See the following figure:

After the program runs, you can see IP list of the decoders currently connected into the network. The product serial numbers of all network video decoders searched in the current LAN, and the current IP address and firmware version would be displayed on the interface. If the network video decoder is not in the same network segment as your PC, visit between them is forbidden. For instance, if your IP address is 192.168.100.33, and the subnet mask is 255.255.255.0, then the IP address interval that can be visited by your PC is 192.168.100.1~192.168.100.255. If the IP address of network video decoder is not within this range, it can not be visited by the PC directly. In this case, you can click “IP Address Setting” to set up the selected network video decoder.

You can also use decoder Matrix World software to search settings, figure as follows:

Note: After revision of the parameters, please restart the device to enable new setting.

### 3.4.2 Adding decoder

Run the decoder’s integrated control software named IMagineWorldDMatrix, and the following interface would appear:

Click to open menu under “Operate” on the top menu column, and select “Configure decoder”.

Then a Msg box “Assign decoder” would eject, then click the button “search”, the decoder would be added into the configuration list automatically. Click “Cancel” to close the dialog box. The result of this operation is shown as follows. (Automatic connection would be done after adding the decoder)

Click the button “Search” below to search the decoder; click “Add To” and “OK” to add the decoder.

If you want to add the server manually, you should click the right key in the window “Device”, then click “add project” in the menu ejected, enter the name, then click the right key to the created project, select “Add a group”; then click the right key again to the created group to select “Server Setting”. Then fill in Server Name and Server Address in the Msg box ejected, and also type in the username and password. Click “OK”. See the following figure:

Step: “Device---add project---add a group---server setting---fill in server name and IP address”

Note: After adding the server, you can use mouse’s left key moving on a camera logo and drag it into black window of left or select a channel and double click the “Assign video”; then the video display in left window.

If you want to disconnect the server, you can click the right key in the video window to select “Del View Param”. See the figure below:

Note: 1. If the server is not assigned, “Del View Param” and “Add To Loop List” are invalid.

2.One channel decoder, “view mode” is invalid.

In this case, on the right side of the main interface of the integrated control software named ImagineWorldDMatrix, there would be a tree-like structuring indicating all decoders accepting administration. On the left side of the main interface are current decoders. It should be noted that after successful connection of the integrated control software and the decoder, the above information and operation buttons are visible and available; otherwise, they would be disabled.

### 3.4.3 Decoder Param setting

Click “Operate” -> “Decoder Param Setting”.

[IP Address]: Type in the decoder’s IP address.

[Subnet mask]: Type in the decoder’s Subnet mask.

[Gateway]: refers to gateway address of the network where the decoder is located.

[Physical address]: refers to the decoder’s MAC address. (MAC address is unique. Caution is needed when changing.)

[485 Baud rate]: currently modes of 2400，4800，and 9600 are supported.

[485 Protocol]: refers to the PTZ protocol used by the decoder

[485 Address]: Valid address is 0-255.

[DNS]: refers to the address of the domain name resolution server supplied by ISP.

[DNS server]: IP address of the DDNS server.

[DNS server port]: WEB port of the DDNS server.

[Language]: English and Chinese available.

[Video Standard]: PAL and NTSC patterns available.

[Time Reset Enable]: the decoder’s automatic restart switch. The default is started condition. Only under the started condition the “Restart Interval Time” setting is valid.

[Restart Interval Time]: you can set up the time interval of the next restart of the decoder, the default time unit is “hour”.

Note: After revision of the decoder parameter setting, you should click “Store the setting” to make it valid. Restart is needed when pattern is revised.

### 3.4.4 Decoder Channel Param Setting

Set up the alarm OSD superposition subtitle. Pay attention to the range of x: [0,672] and y: [500,544], and the delay time of channel picture as well; see the following figure:

### 3.4.5 Alarm Param Setting

Set up the alarm parameters: Check the settings under the “Alarm Linkage” after you have checked “Start” to enable; see the following figure:

You can set up whether to start functions such as “Alarm linkage”, “Video Move” and “Video Lost” or not under the specified channel.

- Alarm linkage:

It is the overall enable indication of the whole Alarm Param Setting. When this item is started, the items under it can be enabled.

- Sensor 1:

This setting means the condition control of the output channel of the corresponding decoder in case Sensor 1 on the server connected alarms. There are 8 selection box following, you can select multiple items. When one channel among them is selected, and Sensor 1 alarms, the corresponding channel shall be open. Trough manual clearing alarm, the channel can be turn to “close”. Click on the selection box, when there is a “√”, the corresponding channel shall be open. Click again, when there is not a “√”, the corresponding channel shall be closed.

Sensor2, sensor3, sensor4 settings same like sensor 1.

### 3.4.6 Audio Control and Talk Control

This function can control on/off Audio of the encoder side external audio device.

Select the channel, click right key on the “Audio Control” and “Open”, and “Talk Control” and “Open”, as shown below:

### 3.4.7 Loop View Setting

Click the “Add” button on the bottom of the Msg box of “Loop View Setting”, then a Msg Box named “Server Setting” would appear,

"Name" is a selective item,

“URL” is the IP address of the camera connected

“Channel” is the number of the channel connecting camera with video server;

“Delay Time” means the Loop View display time, and the default time unit is second.

When you finish setting, click “Start”, then Loop View begins. Click “Stop” to end the loop viewing .

You can also click the right key on the video window, and then click “Loop Control Switch” to start or stop the loop.

The specified picture can be displayed in the specified decoder channel in loop viewing. You can add, cancel or modify parameters of loop view displaying here. Click “Delete Selected Item” to delete the record you have selected; click “Delete all” to delete all channels.

### 3.4.8 PTZ control

You can control camera PTZ via an assigned decoder channel. The PTZ control panel is shown as follows:

Press the left icon on the first row, Server-end PTZ focus will increase; when you stop pressing, such increasing stops.

- Speed adjustment: press number keys 9-0. The corresponding speed is 1-10.

- Left: press [←]. The Server-end PTZ would turn left. Release, it would stop.

- Right: press [→]. The Server-end PTZ would turn right. Release, it would stop.

- Up: press [↑]. The Server-end PTZ would turn up. Release, it would stop.

- Down: press [↓]. The Server-end PTZ would turn down. Release, it would stop.

- Focus +: press [ESC]. The Server-end PTZ focus would increase. Release, it would stop.

- Focus -: press [Edit]. The Server-end PTZ focus would reduce. Release, it would stop.

- Aperture +: press [MENU]. The Server-end PTZ focus would increase. Release, it would stop.

- Aperture -: press [ABC]. The Server-end PTZ focus would reduce. Release, it would stop.

- Zoom +: press [+]. The Server-end PTZ zoom would increase. Release, it would stop.

- Zoom -: press [-]. The Server-end PTZ zoom would reduce. Release, it would stop.

- Press [OK]. Return to the PTZ control interface.

### 3.4.9 Tools

1. Search Decoder: click “Search Decoder” in the menu “”, the following menu would appear:

Click “Modify”: a dialogue box named “Network Param Modify” would appear. See the following picture:

Click “Reset” to reset the device.

Note: you have to reset the device to enable new settings after you have modified the parameters.

2. System Log: click “System Log” to display decoder's disconnection and connection info, see the following figure:

### 3.4.10 Use the software IMagineWordDMatrix for upgrading

Install and operate the IMagineWordDMatrix that is specialized for decoders. Open [Decoder] menu, select [Decoder Setting], the operations are as follows:

Select 	[Decoder Setting], then a dialog box – [Setting Param] would appear as follows:

Select [Upgrade Program] in the dialog box - [Setting Param], then a dialog box – [Upgrade Logo] would appear.

Click [Browse] in the dialog box – [Upgrade Logo], and select a Logo file, click [OK] to start upgrading. After successful upgrading, “Successful Upgrading” would appear in the [status] prompt. The decoder would be restarted automatically. Click “exit” after the upgrading finishes.

## 3.5 Operation of dummy keyboard ( Remote Controller)

### 3.5.1 Instructions of main menu

Press [Menu] to type in main menu as follows:

[View On]: enter connecting server menu

[View Off]: disconnect from the server

[Loop View]: enter loop connection menu

[Loop Cfg]: enter the loop setting menu

[Play Audio]: open the audio output

[View Mode]: close the audio output

[Talk On]: start Talk to the server

[Talk Off]: stop Talk to the server

[Settings]: enter the system setting menu

[PTZ CTRL]: enter PTZ control menu

[Alarm Status]: enter alarm status and control menu

[Alarm Settings]: enter alarm setting menu

Before connecting to the server, buttons such as [View Off], [Play Audio], [View Mode], [Talk On], [Talk Off], and [PTZ CTRL] are unavailable. After connecting to the server, [View On] is not available.

### 3.5.2 View mode and view mode interface

You can select view mode of single picture or mode of four pictures.

### 3.5.3 View On menu

Select [View On] menu on the operation main menu interface, press [OK] to type in [View On] operation interface; see Figure 4.6.3. This interface is used for setting parameters of the server being connected. Press [ESC] or [MENU] to return to the main menu interface.

URL：

Move the Msg Box to “URL”, press [edit] to enter editing status, press number keys to type in IP address, press [+] to type “.”. Or press [ABC] to shift to character input to type in domain name of the server. After typing, press [OK] to confirm, or press [ESC] to cancel. The length is 1-30 bits.

Channel:

Channel is connecting to the server. Move the Msg Box to “Channel”, directly press the number keys to type in channel number or press [+] and [-] to select the channel.

- Transmission methods: select audio-video data transmission protocol. Available protocols include UDP protocol, multicast protocol and TCP protocol. Move the Msg Box to “Transmission methods”, press [+] and [-] to select. In case WAN is used to connect to the server, you need to select TCP protocol to connect to the server.

- Port: The port number is corresponds to the encoder port. Move the Msg Box to “Port”. Press [edit] to enter editing status, press number keys to type in port number, and press [OK] to confirm, or press [ESC] to cancel.

- Username:

It refers to username examined by the server. Move the Msg Box to “username”. Press [edit] to enter editing status; you can press number keys and characters, and press [ABC] to shift input methods. The length is 1-8 bits.

- Password: It refers to password examined by the server. Move the Msg Box to “username”. Press [edit] to enter editing status, press number keys and characters, and press [ABC] to shift input methods. The length is 1-8 bits.

- Auto Connect: Set up “start” to connect the server or to implement repetition connection.

- After using function of automatic connection of the server, the device would connect to the server automatically after the start-up finishes. After using function of automatic repetition connection of the server, the device would implement repetition connection as per settings after the start-up finishes. Move the Msg Box to “Start” to select.

- Dec Ch: if you need to select a server from the Dec Ch, you can move the Msg Box to “Dec Ch”, press [OK] to type in “Dec Ch” menu. (This function is not realized.)

- Dynamic Domain Name: dynamic domain name (DDNS) is needed for connection to server. Set up this item, and type in the server name.

- Talk : You can select if “Talk” shall be started when connecting,

### 3.5.4 Address book menu. (This function is not realized yet.)

### 3.5.5 Loop connect menu

Select "Loop Connect” on the main menu interface, and press [OK] to type in "Loop Connect” menu interface. Move the Msg Box to “start”, press [OK] to start "Loop Connect”. Move the Msg Box to “stop”, press [OK] to stop "Loop Connect”.

Note: Please confirm you have configured "Loop Connect” before that.

### 3.5.6 Loop setting menu

Select "Loop Config" on the main menu interface, and press [OK] to type in "Loop Config" menu interface. See Figure as following. This menu can help finish loop server settings.

Move the Msg Box to the list box, press [OK] to type in “Edit Server” menu interface, or press [edit] to select server. Move the Msg Box to “Edit’, press [OK] to type in “Edit Server” menu interface. Move the Msg Box to “Cancel”, press [OK] to delete the server. Interface of server editing is shown as follows:

### 3.5.7 Interface of Add Server

On the menu interface of loop connection setting, move the Msg Box to “Add”, press [OK] to enter this menu. This menu can help finishing loop server settings.

- URL: Move the Msg Box to “URL”, press [edit] to enter editing status; press number keys to type in IP address, press [+] to type in “.”. Press [ABC] to shift input. You can enter the domain name of the server. The length is 1-30 bits.

- Port: move the Msg Box to "port", press [edit] to enter editing status, press “OK” to input port number,

- Channel: move the Msg Box to “Channel”. Press number keys to input channel number or press [+] and [-] to select channel number.

- Transmission protocol: move the Msg Box to “Protocol”, press [+] and [-] to select. Available protocols include UDP protocol, multicast protocol and TCP protocol.

- Username: Move the Msg Box to “username”. Press [edit] to enter editing status, press number keys and characters. The length is 1-8 bits.

- Password: Move the Msg Box to “Password”. Press [edit] to enter editing status, press number keys and characters. The length is 1-8 bits.

- Loop view Time : move the Msg Box to “Loop View Time”; press [edit] to enter editing status, press number keys to input staying duration, within the range - 5-1800 seconds.

### 3.5.8 Start and stop the audio

- Move the Msg Box to "start the audio" on the main menu interface, select channel, click [OK] to start the audio. Click again to stop the audio.

### 3.5.9 Start and stop Talk

- Move the Msg Box to "Start Talk” on the main menu interface, select channel, and click [OK] to start Talk. Click it again to stop Talk.

### 3.5.10 System setting menu interface

On the main menu interface, move the Msg Box to “Settings”, press [OK] to enter this menu.

- Network: enter “network setting” menu interface

- Param: enter “parameter setting” menu interface

- VGA Param: enter “Video Param” menu interface

- Audio Param: enter “Audio Param” setting interface

- Addr List: enter “Addr List” menu interface. (This function is not realized yet.)

- Version: enter “software version information” menu interface

- Save: store the parameters permanently

- Reset: restart the server

### 3.5.11 Network setting menu interface

- On the setting menu interface, move the Msg Box to “Network”, press [OK] to enter this menu. See Figure 4.6.11. This menu can help finishing settings of the network components such as IP address, DNS, and so on.

- IP address: Move the Msg Box to “IP address”, press [edit] to enter editing status, press number keys to type in IP address, press [+] to type in “.”. Or press [ABC] to shift input methods to type in the domain name of the server. The length is 1-30 bits. Press [Esc] to return to “setting menu”.

- Subnet mask: Move the Msg Box to “subnet mask”, press [edit] to enter editing status, press number keys to type in, press [←] and [→] to move the range.

- Default gateway: Move the Msg Box to “default gateway”, press [edit] to enter editing status, press number keys to type in, press [←] and [→] to move the range.

- DNS：

Move the Msg Box to “DNS”, press [edit] to enter editing status, press number keys to type in, press [←] and [→] to move the range.

- DNS setting: set up DNS and DDNS in the following picture first; and the port shall be set up in the connection menu.

- Name: refers to registered device name

- URL: refers to DNS server’s address

- Using DDNS

- Port: refers to port of the DNS server. See the following figure:

### 3.5.12 System Param menu interface

On the setting menu interface, move the Msg Box to “System Param”, press [OK] to enter this menu,. See the figure:

- VO Standard: move the Msg Box to “VO Standard”. Press [+] and [-] to select VO Standard. The available patterns are: PAL and NTSC.

- Language: move the Msg Box to “language”, press [+] and [-] to select. The available ones are English and Chinese. After changing the language of the interface, the main menu would change too.

- Protocol: in case of using keyboard control decoder of the controlling globe camera, RS485 protocol, RS485 baud rate and RS 485 address shall apply. Move the Msg Box to “protocol”, press [+] and [-] to select.

- Baud rate: move the Msg Box to “baud rate”, press [+] and [-] to select. Currently, 2400, 4800, and 9600 are supported.

- RS485 address: move the Msg Box to “baud rate”, press number keys to input address or press [+] and [-] to select. Press [edit] to delete. The valid address is 0-255.

- RS 485 function: move the Msg Box to “RS485”, press [+] and [-] to select the functions, keyboard and transparent serial port to be realized. The keyboard function means to control the menu through realizing external keyboard. The transparent serial port function means to realize alarm control or RS485 outputs corresponding signals in case of linkage, which shall be determined by the protocol.

### 3.5.13 Video Param Interface  (VGA Param)

- Move the dialog to VGA Format, press [+] and [-] to select the format. The available range include 800*600_50HZ, 1024*768_60HZ, 1280*720_60HZ, and 1920*1080_60HZ.

Note: if the resolution of monitor is lower than the resolution you have set up, input error will display.

### 3.5.14 Audio Param setting interface

- Move the Msg Box to “Audio Input” , press [+] and [-] , MIC Input and Line Input for optional .

- Set ”Line Input Volume”; press [Edit ] to enter editing status. Press number keys to input. Press [←] and [→] to move the range.

- Move the Msg Box to “Mic Boost”, ON/OFF, select open and close microphone plus.

- Move the Msg Box to “Audio Out Volume” to control Decoder output volume. Press [Edit] to enter editing status. Press number keys to input. Press [←] and [→] to move the range.

### 3.5.15 PTZ control menu

On the main menu interface, move the Msg Box to “PTZ CTRL”; press [OK] to enter this menu. See Figure 4.6.15. Press [ESC] or [MENU] to return to main menu. (The alarm auxiliary devices and alarm output functions are not realized yet.)

- Preset position: move the Msg Box to “Ch 1”, press the number keys or press [+] and [-] to select. Press [Edit] to delete. The valid address is 0-128. Move the Msg Box to “preset”, press [OK] to preset. Move the Msg Box to “transfer”; press [OK] to transfer.

- Auxiliary device: move the Msg Box to “Device”, press [OK] to open device; and then press [OK] to close device . (This function is not realized yet.)

- Alarm output: move the Msg Box to “alarm output”, press [OK] to open the alarm output; and then press [OK] to close alarm output. (This function is not realized yet.)

- Auto: move the Msg Box to “Auto”. Press [OK] to start Dome Camera automatic rotation function; the button would turn to red. Press [OK] again stop automatic rotation function; the red button turn back to green color.

- PTZ Control: move the Msg Box to “PTZ Control”, press [OK] to type in PTZ Control interface. After entering PTZ control interface, the menu would hide. Press the following keys to control PTZ:

- Speed adjustment: press number keys 9-0. The corresponding speed is 1-10.

- Left: press [←]. The Server-end PTZ would turn left. Release, it would stop.

- Right: press [→]. The Server-end PTZ would turn right. Release, it would stop.

- Up: press [↑]. The Server-end PTZ would turn up. Release, it would stop.

- Down: press [↓]. The Server-end PTZ would turn down. Release, it would stop.

- Focus +: press [ESC]. The Server-end PTZ focus would increase. Release, it would stop.

- Focus -: press [Edit]. The Server-end PTZ focus would reduce. Release, it would stop.

- Aperture +: press [MENU]. The Server-end PTZ focus would increase. Release, it would stop.

- Aperture -: press [ABC]. The Server-end PTZ’s focus would reduce. Release, it would stop.

- Zoom +: press [+]. The Server-end PTZ’s zoom would increase. Release, it would stop.

- Zoom -: press [-]. The Server-end PTZ’s zoom would reduce. Release, it would stop.

- Press [OK]. Return to the PTZ control interface.

### 3.5.16 Alarm status menu interface

On the main menu interface, move the Msg Box to “alarm status”. Press [OK] to enter this menu. See Figure 4.6.16.

Press [ESC] or [MENU] to return to the main menu. The control of the four alarm channels output and connected Server-end’s alarm status are to be realized. The sensor, camera moving and video lost buttons are in read-only status.

(Note: you need to set up the transparent serial port enable)

- Sensor:

This item displays the sensor’s alarm status on the server connected. There are four sensor statuses. In case the button’s color is green, there would be no alarm. In case the button’s color is red, there would be alarm.

- Video move:

This item displays the Video Move alarm status on the server connected. In case the button’s color is green, there would be no alarm. In case the button’s color is red, there would be alarm.

- Video lose:

This item displays the video lose alarm status on the server connected. In case the button’s color is green, there would be no alarm. In case the button’s color is red, there would be alarm.

- Clear the alarm

This item is used for clearing the alarm status on the server. Press [OK] to realize one-time clearing order.

### 3.5.17 Alarm setting menu interface

On the main menu interface, move the Msg Box to “alarm setting”. Press [OK] to enter this menu.

Press [ESC] or [MENU] to return to the main menu. This menu is mainly for realization of the linkage control setting of the four route alarm output and input.

- Alarm linkage:

It is the overall enable indication of the whole alarm setting. When this item starts, the settings under it would become valid. Press [+] and [-] to set the status to “enable” or “disable”. Among this, the linkage blow up is invalid due to single channel.

- Sensor  1:

This setting means the condition control of the output channel of the corresponding decoder in case Sensor 1 on the server connected alarms. There are 4 selection box following. You can select multiple items. When one channel among them is selected, and Sensor 1 alarms, the corresponding channel shall be open. Trough manual clearing alarm, the channel can be turn to “close”. Press [OK], when there is a “√”, the corresponding channel shall be open. Press [OK] again, when there is not a “√”, the corresponding channel shall be closed.

- Frequent Asked Questions

  - Fail to connect the server

Check the network wire is connected correctly and IP address and port numbers are correct or not. In addition, check the local IP address, subnet mask, default gateway and DNS are set correctly or not.

  - Fail to connect the server through the wide area network

Firstly, check the network wire is connected correctly and IP address and port numbers are correct or not. In addition, check the default gateway and DNS in the “network setup” are set correctly or not.

  - The dome camera fails to control the decoder

Check the RS485 connection is connected correctly or not and the settings of RS485 address, RS485 protocol and RS485 baud rate are matched with the keyboard or not.

  - Fail to control the cradle head on the server end

Confirm that the cradle head on the server end is connected correctly or not and please refer to the server instruction manual for details.

- Appendix Dome Camera Keyboard Application

1 Channel MPEG-4 embedded type decoder supports the dome camera keyboard control. At present, it supports PELCO-P (baud rate 2400, 4800 and 9600) protocols. The operation of the keyboard corresponding to the keys of remote controller:

【↑】：Up

【↓】：Down

【←】：Left

【→】：Right

【Edit】：IRIS –

【Enter】：IRIS +

【+】：FOCUS +

【-】：FOCUS –

【Abc】：ZOOM +

【ESC】：ZOOM –

【MENU】：Preset Point Call

In the interface of cradle head control, press the calling+65 to exit the cradle head control interface.

Thanks for using this network video decoder. Owing to the differences between each model or appearance or function differing from those described in this manual because of system software updating, please contact the manufacturer at any time. Please forgive that we could not inform you of these changes timely.