Network Video Decoder

Centralized Management Software

MatrixWorld

(Ver1.1)

User Manual

Update Time: 2008-2-27

Contents

Introduction	2

Parameters:	3

Appearance	4

Operations:	6

Introduction to MatrixWorld Decoder Control Software	6

Installation of MatrixWorld	6

Start MatrixWorld	7

Top Menu	8

Files	8

Operations	9

Decoder	9

Tools	10

Help	10

Distribute CCD Camera	10

Decoder Parameters Setting	13

Alarm Parameters Setting	14

Virtual Keyboard	16

Transparent Com	16

Search Decoder	16

Operation Instruction for MatrixWorld Principal Interface	17

Software Upgrading for 8000E Series Built-in Decoder	20

Questions and Answers:	21

# Introduction

Thank you very much for buying our products. We will provide you with best services!

With the development of computer and information network technology, security protection monitoring technology is also developed to be digitalized and network. Digitalized and network monitoring system has stepped into complete digitalized age, destroyed simulation mode of “CCTV Monitoring System” thoroughly, and changed the way and structure of CCTV system from information collection, transmission and processing to system controlling.

Our self-developed 8000E series built-in decoder is one series of Digital Network Products. The main function is to reduce the audio code data of 9000 series and 8000E series video server transmitted by network back to analogue AV signals, which will be displayed on monitor. It is applicable to controlling center with large or medium sized monitoring system.

Functions:

- Built-in design with small volume, high integrated level, easy to   organize the network;

- Able to reduce audio code signals exported by 9000 series and 8000E series;

- Possess two independent audio-video output paths, and export the audio-video signals of two paths simultaneously; applicable to many popular matrix host computer;

- Possess one audio output interface, which can realize intercommunication with the video server of forepart;

- 4 different IP address can be displayed on each video path to realize multi-picture display on one monitor;

- Four alarm output interfaces to response alarm information sent by the video server of forepart;

- Self-contained MatrixWorld controlling software by which the control on system and video camera of forepart is able to be realized;

- Support completely transparent RS232 and RS485 interface.

# Parameters:

| Video system | Support NTSC or PAL format |
| --- | --- |
| Audio compression standard | ISO-MPEG Audio Layer-3 |
| averaged bandwidth occupied by video flow | 200 kbps (resolution factor 352×288, 25) |
| Image delay | < 200 ms (LAN) |
| Supported resolution factor | PAL: 704×576, 704×288, 352×288 NTSC: 704×480, 704×240, 352×240 |
| Audio input (selectable) | BNC interface mic. Input or linear input |
| Audio output | BNC interface linear input |
| Hardware | CPU: DSP processor with high performance; Operation system: real-time built-in Vxworks operation system |
| Video output mode | Analogue video output |
| Video system | Support NTSC or PAL format |
| Video output mode | Analogue video output |
| Video decode size | Support D1: 704×576 optimum, compatible Half D1: 704×288 CIF: 352×288 |
| audio | Double-way audio intercommunication |
| Video decode format | Support code data of MPEG-4 video server |
| Built-in GUI | visual and convenient operation |
| Input method support | Convenient for inputting domain name |
| Image delay | < 200 ms (LAN) |
| multi-server polling | Support path selecting and able to control path patrolling by program |
| Network interface | 10/100M |
| working temperature | 5 ~ 60 ℃ |
| Working humidity | 20 ~ 80% |
| Input voltage | DC12V /3A |
| Power | < 10 W |
| PCB size | 110mm(W) * 70mm(H)*2mm(D)(1/2 bit) |
| Size of complete machine | 205mm(W)*122mm(H)*48mm(D) |

# Appearance

1. Size: 262(W)mm*140(D)mm*43(H)mm

Top view of decoder

2. Front panel:

Front panel of decoder

- USB interface: not used for this edition;

- TX/RX: transmitting indication for control data;

- LNK: network working status indication;

- PWR: power indicator;

- RST: system reset, reset on stage setting. Nonprofessional persons are forbidden to press this button.

3.  Back panel:

Back panel of decoder

- DC12V: power supply socket, matching DC12V power supply;

- LAN: network interface;

- RS232 interface: adjustment interface

- ALARMIN: not used for this version;

- RS485 interface: not used for this version;

- ALARMOUT: alarm output port, 4 groups of controlling relay output contact;

- NORM/SYS alternation switch：alternation switch is located at SYS during adjusting computer by RS232 while it is usually at NORM;

- LINE/MIC alternation switch: electrical level electrical level for intercom; the intercom is line input at LINE while at MIC it is microphone input;

- VOUT2: 2-way video output interface;

- AOUT2: 2-way audio output interface;

- VOUT1: 1-way video output interface;

- AOUT1: 1-way audio output interface;

- AIN: intercom input interface;

4. Wiring Sketch Map

# Operations:

The controlling and setting of 8000E series decoder are all realized by MatrixWorld decoder control software.

# Introduction to MatrixWorld Decoder Control Software

MattrixWorld decoder control software is developed for 8000E series network video decoder. By using this software, operations of setting, definition and control can be done on 8000ER. It is a necessary management tool for 8000ER.

# Installation of MatrixWorld

- Working environment:

Operating system: Windows2000server/professinal/XP

Network protocol: TCP/IP

PC at client side: P4/128MRAM/40GHD/ support zoom display cards, support DirectX7.0 and above

- Installation Guide:

Open the folder in management software and run (double click left button of mouse) installation program file “MatrixWorld_V02.00.00.03.exe”, then the following dialogue-box will appear:

Next, install as per the prompt of installation tool until steps showed as follows appear. Choose [Finish] to complete the installation.

# Start MatrixWorld

Choose program groups for installation in [Start] menu, which is showed as following:

Run MatrixWorld by click the left button of the mouse, and interface as following will appear:

## Top Menu

Top menu mainly contains following functions for option:

## Files

Click “File (F)” option to open sub-menu as following:

Save Configuration: current configuration parameters can be saved as files in local computer.

Reload Config : current configuration parameters can be loaded to current operation.

Exit: close MatrixWorld program.

## Operations

Click “Operation (O)” option to open sub-menu as following:

Edit Decoder: operations like configuring principal/subordinate output modes and adding/deleting an selected decoder can be done on 8000E series decoder.

## Decoder

Click “Decoder (D)” option to open sub-menu as following:

Connect Decoder: perform connecting operation on selected decoders;

Disconnect Decoder: perform disconnecting operation on appointed decoders;

Decoder setting: setting parameters for selected decoders, such as setting IP address, mask coder of sub-network, video system, etc.;

Alarm Setting: setting alarm parameters for selected decoders, and there are functions like alarm interlock, video shift, video loss for option;

Virtual Keyboard: controlling by virtual keyboard in computer;

Transparent serial: RS485 will export corresponding controlling signals during realizing alarm controlling or interlock, which will be determined by protocol.

## Tools

Click “Tools (T)” option to open sub-menu as following:

Decoder search : able to search currently usable decoder, which will be listed automatically.

## Help

Click “Help (H)” option to open sub-menu as following:

The version and copyright information will be displayed in “Help” menu.

## Distribute CCD Camera

Choose 【Configure Decoder】button in “Operations” in top menu.

Click right button of the mouse, and choose “Add Projects” in popup menu. Choose “Add Groups” for newly-built project by clicking right button after entering the description;

Then click on newly-built group to choose “Add Decoder”;

Enter decoder’s name and address, users name and password in popped up dialogue-box. Click to confirm.

- Decoder’s Name: refers to the name of the decoder to be connected. This name is sole ID within same area, so it is required to avoid repetition during registration and connection;

- Decoder’s Address: IP address or domain name can be used as connecting server address, or click “Search and List ” directly, then pop up following dialogue-box:

- Choose “Yes (Y)” to delete the decoder information configured manually and list currently usable decoder’s information and configure automatically instead;

- Choose “No (N)” to cancel automatic searching configuration and return back to current operating interface.

Advice: nonprofessional person can choose “Yes (Y)” for more convenient configuration.

Click “Return” to return following interface:

Click by left button on newly-built decoder, and choose “Connect Decoder” in “Decoder” in top menu.

Note: functions like “set decoder”, “set alarm parameter”, “virtual keyboard” and “transparent com” cannot be used if “connect decoder” isn’t chosen!

# Decoder Parameters Setting

Choose “decoder setting” in “decoder” in top menu, then dialogue-box as following will pop up:

- IP address: enter the IP address of the decoder

- Sub-net code: enter the sub-network mask code of the decoder

- 485 baud rate: currently support three modes like 2400, 4800, 9600

- 485 address: the valid address is 0-255

- Video pattern : there are PAL and NTSC for option

- Restart server: default to be starting status. The setting on time interval for restart will be valid only under starting status

- Restart interval: time interval for next start of decoder can be set here, and the time unit is defaulted as “hour”

Note: the modified decoder parameters will come into effect only after clicking “Save”.

# Alarm Parameters Setting

Choose “alarm parameters setting” in “decoder” in top menu, then dialogue-box as following will pop up:

It is able to set whether to activate the functions of “alarm interlock”, “video shift” and “video loss” in appointed pathway.

- Alarm enable:

It is enable mark for entire alarm setting. Other settings will become valid only after activate this function.

- Sensor 1:

It is status controlling on corresponding output pathway of the decoder if it has been connected with 1# detector alarm on the server. There are 8 choice boxes and multi-choice is allowed. Once one pathway is chosen, corresponding pathway will be opened when 1# detector alarms. The status of this pathway can be changed into close status by resetting alarm manually. Click the choice box once, a “√” will appear in the box, and the status of corresponding pathway will be showed as open. Click again, the “√” will disappear in the box, and the status of corresponding pathway will be showed as close.

- Sensor 2:

It is status controlling on corresponding output pathway of the decoder if it has been connected with 2# detector alarm on the server. There are 8 choice boxes and multi-choice is allowed. Once one pathway is chosen, corresponding pathway will be opened when 2# detector alarms. The status of this pathway can be changed into close status by resetting alarm manually. Click the choice box once, a “√” will appear in the box, and the status of corresponding pathway will be showed as open. Click once again, the “√” will disappear in the box, and the status of corresponding pathway will be showed as close.

- Sensor 3:

It is status controlling on corresponding output pathway of the decoder if it has been connected with 3# detector alarm on the server. There are 8 choice boxes and multi-choice is allowed. Once one pathway is chosen, corresponding pathway will be opened when 3# detector alarms. The status of this pathway can be changed into close status by resetting alarm manually. Click the choice box once, a “√” will appear in the box, and the status of corresponding pathway will be showed as open. Click once again, the “√” will disappear in the box, and the status of corresponding pathway will be showed as close.

- Sensor 4:

It is status controlling on corresponding output pathway of the decoder if it has been connected with 4# detector alarm on the server. There are 8 choice boxes and multi-choice is allowed. Once one pathway is chosen, corresponding pathway will be opened when 4# detector alarms. The status of this pathway can be changed into close status by resetting alarm manually. Click the choice box once, a “√” will appear in the box, and the status of corresponding pathway will be showed as open. Click once again, the “√” will disappear in the box, and the status of corresponding pathway will be showed as close.

- Video Move:

It is status controlling on corresponding output pathway of the decoder when there is video shift alarm showed in the connected server. Click the choice box once, a “√” will appear in the box, and the status of corresponding pathway will be showed as open. Click once again, the “√”in the box will disappear, and the status of corresponding pathway will be showed as close.

- Video Loss:

It is status controlling on corresponding output pathway of the decoder when there is video loss alarm showed in the connected server. Click the choice box once, a “√” will appear in the box, and the status of corresponding pathway will be showed as open. Click once again, the “√”in the box will disappear, and the status of corresponding pathway will be showed as close.

# Virtual Keyboard

MatrixWorld software provides virtual keyboard function, which can simulate really keyboard to complete the controlling.

# Transparent Serial

The function of transparent com is to realize alarm controlling or export corresponding signals by RS485 during interlock, which will be determined by the protocol. It can use com to transmit data.

# Search Decoder

Choose “search decoder” in “Tools” in top menu.

Click “search” in popped-up dialogue-box, then the information of currently usable decoder will be listed out.

Click the decoder to modify the decoder’s name, IP address and sub-network mask code, which is showed as follow picture:

# Operation Instruction for MatrixWorld Principal Interface

- Output mode: that output images will be displayed by which modes can be set here. There are two modes for option: single image, 4 images.

- Current channel: by choosing this option, current output mode will be defaulted as appointed pathway which is displayed in mode of single image.

- Decoding channel: the status of current decoding pathway can be controlled here.

Note: when it is necessary to set up new connection, current decoding pathway have to be disconnected, then new connection set-up can be proceeded.

- Loop setting: the selected images shot by CCD camera can be displayed repeatedly in appointed decoding pathway, and parameters of polling can be added, reduced and modified here. Please check following picture:

“Name” is for option, “URL address” is the IP address of the CCD camera that have been connected, “pathway” is pathway for CCD camera that connects with video server, “dwell time” refers dwell time of the images and the unit is defaulted as second.

After finishing setting, click “start” to start polling, and click “stop” to stop polling.

“Cradle head control”: cradle head can be controlled by appointed decoding pathway.

FOCUS+: press 【+】 button to control the focus + of the cradle head at server end, release the button to stop;

FOCUS-: press 【-】 button to control the focus - of the cradle head at server end, release the button to stop;

IRIS+: press 【+】 button to control the iris + of the cradle head at server end, release the button to stop;

IRIS-: press 【-】 button to control the iris - of the cradle head at server end, release the button to stop;

ZOOM+: press 【+】 button to control the zoom + of the cradle head at server end, release the button to stop;

ZOOM-: press 【-】 button to control the zoom - of the cradle head at server end, release the button to stop;

Preset: set preset point with max. quantity of 10;

Call: recall to preset points;

Left: press 【←】, the cradle head at server end will turn left, release the button to stop;

Right: press【→】, the cradle head at server end will turn right, release the button to stop;

Up: press【↑】, the cradle head at server end will turn upward, release the button to stop;

Down: press【↓】, the cradle head at server end will turn downward, release the button to stop;

Auto: press【auto】, the cradle head at server end will automatically rotate horizontally, release the button to stop;

Return: press【return】 to go back to principal controlling interface.

# Software Upgrading for 8000E Series Built-in Decoder

Click “set decoder” in “decoder” in top menu. Software updating operation can be achieved at the bottom of the popped up dialogue-box.

Interface of upgrading is showed as the picture:

- Upgrading program

- Upgrading LOGO:

- Upgrading Protocol:

# Questions and Answers:

- unable to connect with the server

Check and confirm if the connection is correct first, and then check if IP address and port number is correct. Further more check whether settings of local IP address, sub-network mask, default gateway and DNS in “Network Setting” are correct or not.

- Unable to connect the server by wide area network

Check and confirm if the connection is correct first, and then check if entered domain name and port number is correct. Further more check whether settings of default gateway and DNS in “Network Setting” are correct or not.

- The keyboard of spherical camera cannot control decoder

Check if RS485 connection is correct; check whether settings for RS485address, agreement and baud rate can match the keyboard.

- Unable to control the cradle head at server end

Check to confirm if the connection of cradle head at server end is correct. Please refer Users’ Manual of Server for details.